# Chapter 2.2

Given:

- L1 cache latency: 1 cycle

- DRAM latency: 100 cycles

- Cache line size: 4 words

- Processor frequency: 1 GHz

- Dot product loop: dot_prod += a[i] * b[i]

Each iteration requires:

- load a[i]

- load b[i]

- multiply

- add

A DRAM cache line brings in 4 elements of a or 4 elements of b.

For every 4 iterations:

- 1 DRAM miss for a → 100 cycles

- 1 DRAM miss for b → 100 cycles

Total = 200 cycles for 4 iterations

Each iteration performs:

- 1 multiply

- 1 add → 2 FLOPs

Total FLOPs for 4 iterations:

4 iterations × 2 FLOPs = 8 FLOPs

Performance:

8 FLOPs / 200 cycles

At 1 GHz → 200 cycles = 200 ns

GFLOPs = 8 / (200 × 10^-9)

GFLOPs = 40 × 10^6 FLOPs

Peak performance = 40 MFLOPs = 0.04 GFLOPs

Chapter 2.9

The CRCW PRAM model is the best.

You can see this in the rules for accessing memory.

EREW: two processors can't read or write to the same place at the same time.

CREW: many reads are okay, but only one write is okay.

ERCW: one read, and many writes are allowed

CRCW: allows more than one read and write

CRCW has the most parallelism since all the processors can read and write to the same memory cell at the same time.

With CRCW, problems can be solved quickly, which is not possible with weaker models.

For example, you may do a parallel reduction in O(1) time by using concurrent writing and a priority control.